✅ Project 1: Auto Water Dispenser

Components:

1 Ultrasonic Sensor (HC-SR04)

1 Relay Module (to control Air Pump)

1 Air Pump

✅ Project 2: Auto Opening Dustbin

Components:

1 Ultrasonic Sensor (HC-SR04)

1 Servo Motor (e.g., SG90)

🔌 Wiring Connections

🟦 Ultrasonic Sensor for Water Dispenser

Ultrasonic Pin	Arduino Pin

VCC	5V

GND	GND

TRIG	2

ECHO	3

🟩 Relay Module for Air Pump

Relay Pin	Arduino Pin

IN	4

VCC	5V

GND	GND

🟦 Ultrasonic Sensor for Dustbin

Ultrasonic Pin	Arduino Pin

VCC	5V

GND	GND

TRIG	6

ECHO	7

🟨 Servo Motor

Servo Pin	Arduino Pin

Signal	9

VCC	5V

GND	GND

✅ Arduino Code for Both Projects

cpp

Copy

Edit

#include <Servo.h>

#define DISP_TRIG 2

#define DISP_ECHO 3

#define RELAY_PIN 4

#define BIN_TRIG 6

#define BIN_ECHO 7

#define SERVO_PIN 9

Servo binServo;

void setup() {

pinMode(DISP_TRIG, OUTPUT);

pinMode(DISP_ECHO, INPUT);

pinMode(RELAY_PIN, OUTPUT);

pinMode(BIN_TRIG, OUTPUT);

pinMode(BIN_ECHO, INPUT);

binServo.attach(SERVO_PIN);

binServo.write(0); // Dustbin closed

digitalWrite(RELAY_PIN, LOW); // Pump OFF

Serial.begin(9600);

}

void loop() {

// --- Water Dispenser ---

long dist1 = getDistance(DISP_TRIG, DISP_ECHO);

if (dist1 < 10) {

digitalWrite(RELAY_PIN, HIGH); // Turn ON Pump

Serial.println("Water Dispensing...");

delay(3000); // Dispense time

digitalWrite(RELAY_PIN, LOW); // Turn OFF Pump

}

// --- Auto Dustbin ---

long dist2 = getDistance(BIN_TRIG, BIN_ECHO);

if (dist2 < 15) {

binServo.write(90); // Open bin

Serial.println("Bin Opened");

delay(3000);

binServo.write(0); // Close bin

Serial.println("Bin Closed");

}

delay(500);

}

// Function to get distance from ultrasonic sensor

long getDistance(int trigPin, int echoPin) {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH);

long distance = duration * 0.034 / 2;

return distance;

}